VLAN & IP Subnet Scoping Framework

A practical one-page guide for building clean, scalable network segmentation in a home lab or production-style environment

Core mentality: treat VLANs and subnets as boundaries for function, security, and troubleshooting — not just as IP ranges.

1. Start with purpose

Create segments based on what the network needs to support, such as management, servers, users, IoT, cameras, guest wireless, storage, or test/lab networks. If two groups require different trust levels or different firewall policy, they usually belong in different VLANs.

| • Management • Servers / services • User devices • Storage / backup | • IoT / smart devices • Cameras / access control • Guest wireless • Lab / testing / DMZ |
| --- | --- |

2. Keep the design predictable

Use a consistent mapping between function, VLAN ID, and subnet so the design is easy to read at a glance.

| VLAN | Function | Example subnet (Site 1) |
| --- | --- | --- |
| 10 | Management | 10.10.10.0/24 |
| 20 | Servers | 10.10.20.0/24 |
| 30 | Users | 10.10.30.0/24 |
| 40 | IoT | 10.10.40.0/24 |
| 50 | Guest | 10.10.50.0/24 |

3. Scale by site, not by reinventing the design

Keep the same VLAN numbers for the same functions across every site and change only the site portion of the address.

| VLAN / Function | Site 1 | Site 2 | Site 3 |
| --- | --- | --- | --- |
| 10 / Mgmt | 10.10.10.0/24 | 10.20.10.0/24 | 10.30.10.0/24 |
| 20 / Servers | 10.10.20.0/24 | 10.20.20.0/24 | 10.30.20.0/24 |
| 30 / Users | 10.10.30.0/24 | 10.20.30.0/24 | 10.30.30.0/24 |
| 40 / IoT | 10.10.40.0/24 | 10.20.40.0/24 | 10.30.40.0/24 |
| 50 / Guest | 10.10.50.0/24 | 10.20.50.0/24 | 10.30.50.0/24 |

4. Build policy and growth into the design

| Firewall mindset • Default deny between VLANs unless there is a reason to allow it • Management should reach infrastructure intentionally • Guest and IoT should be heavily restricted • Document the reason for every allow rule | Design habits that scale • Name VLANs by function, not temporary project names • Reserve extra space for future segments • Keep gateways and DHCP ranges consistent • Make the scheme simple enough that another engineer can guess it |
| --- | --- |

Bottom line: a mature VLAN/subnet design is obvious, predictable, secure, and easy to troubleshoot.